Operation of a Reservoir with Combined use of DP-GA

Deepti Rani*

Department of Hydrology, IIT Roorkee, Roorkee, INDIA,

Email: deeptinatyan@yahoo.com, Tel.: 0091-1332-286561, Fax: 0091-1332-285236

Dinesh Kumar Srivastava

Department of Hydrology, IIT Roorkee, Roorkee, INDIA,

Email: dksfhy@rediffmail.com, Tel.: 0091-1332-285389

Abstract  Dynamic programming (DP) and Genetic algorithms (GAs) optimization techniques have been successfully used for single and multi reservoir operation problems. The use of GAs constrain to try different alternatives, different GA parameter values, and select those which perform best for a particular application. The effect of parameter adjustments is not known until the algorithm terminates. This makes the process extremely time-consuming. The GA starts with randomly generated initial population within the entire search space to reach to optimal. This may be time-consuming and there are chances of getting trapped into local optima too. The GA can perform faster to arrive at the optimal, if the initial population could be defined within the range where optimal solution would exist. This process shall save time spent on setting and selection of parameters. Discrete DP provides global optimal solutions, but discreteness and dimensionality in the problem cause the main disadvantages in reservoir system operations applications. This difficulty may be overcome by a combination of DP and GA (DP-GA) approach. The DP would then provide the range within which the optimal solution for the problem is expected. The GA later would search and achieve the possible optimal solution within this range. This proposed methodology is presented here to study Mula reservoir operation in India. The objective is to achieve an optimal operating policy by minimizing the deficits in meeting water target demands. A comparison of DP-GA and GA shows that, DP-GA outperforms GA and saves the time spent on selecting some of the parameters. Also a faster convergence to the optimal solution is likely to be achieved.

Keywords: Genetic algorithm, dynamic programming, reservoir operation, large size reservoir

- Introduction:

Over the last decade, evolutionary and meta-heuristic algorithms have been extensively used as search and optimization tools in various problem domains. Their applicability to wide-range of problems, easy to use, and global perspective are the main reasons which makes them preferred tools for extensive and successful applications to a variety of problems. GA belongs to the evolutionary algorithms (EA) family which has been used repeatedly in reservoir operation studies during last two decades. The evolutionary algorithms like GA, ACO & PSO have great potential for handling non-linearity and solving multi-objective problems. Genetic algorithm has been applied to many real life optimization problems by several researchers.  GAs are search and optimization procedures that are motivated by the principle of natural genetics and natural selection [1-7].

Traditional GA or simple GA along with many attractive features poses computational difficulties, e.g., convergence speed to attain the optimum and getting stuck at local optima. Therefore, researchers have made efforts to modify GA itself or used it with other techniques to overcome these drawbacks. The main drawbacks are slow convergence, sensitivity analysis to find proper GA parameters so as to reach the optimum and getting struck into local optima, which is usually due to the initial population chosen or the parameters chosen. Ganji et al. [8] developed a modified version of the simple GA, called SGA, for application to a reservoir operation problem. The SGA reduces the overall run time as compared to the simple GA through dynamically updating the length of chromosomes. Karamouz et al. [9] solved a similar problem using a GA-K nearest neighborhood based optimization model. In this methodology, the lengths of chromosomes are increased based on the results of a KNN forecasting model. A hypercubic distributed GA (HDGA) was applied by Chen and Chang [10] with three general GA operators, linear ranking selection, blend-α crossover and Gaussian mutation, to search for the optimal reservoir release.

Recently, Hinçal et al. [11] applied GA to a multireservoir system operation to maximize the energy production in the system by using two different approaches, conventional (monthly) and real-time approach. The comparison of the results revealed that the energy amounts optimized by using conventional approach were higher than the energy produced in real time operation. However, by using real-time approach, a close approximation to the real operational data had been achieved.

The immune genetic algorithm (IGA), which is based on the immune network theory and GA, was introduced in the reservoir optimization field by Wensheng et al. [12] as an adaptive immune GA (AIGA) for finding optimal reservoir dispatching. The main features of AIGA are the automatic changing of the crossover probability and mutation probability with respect to the fitness value of the objective function. A noisy GA (NGA) was proposed by Yun et al. [13] to deal stochastic inflow pattern. They compared the efficiency of NGA with Monte Carlo method and concluded that the NGA shows better performance than the Monte Carlo optimization technique.

Generation of random initial population plays an important role in faster convergence and to reach global optimal solution for GA and simple GA often suffers when initial random population is not so good. To increase the convergence speed of GA and to overcome getting stuck at local optima, Cheng et al. [14] used a chaos GA (CGA) in the optimization of a hydropower generating reservoir system. The authors presented a CGA on the basis of integrating the GA and the chaos optimization algorithm (COA).

It is reported in a recent review paper that combined or hybrid of GAs techniques showed better results than simply GA [15]. Rani and Moreira [16] reviewed both traditional and computational intelligence techniques used in reservoir operation studies they also ended up giving emphasis to investigate combined use of traditional and computational intelligence approaches in order to explore the unique advantages of these techniques.

Cai et al. [17] presented a combined GA-LP strategy to solve large nonlinear reservoir systems optimization model. GA was used to linearize the original problem in each time period which is later solved sequentially using LP. The hybrid GA-LP approach was capable to find good approximate solutions to the nonlinear models. In view of computational advantages of combined GA-LP strategies to deal with nonlinearities, Reis et al. [18] proposed and evaluated a stochastic hybrid GA-LP approach to operation of reservoir systems, which admits a variety of future inflow variability through a treelike structure of synthetically generated inflows.

Huang et al. [19] presented a GA-based SDP model to cope with the dimensionality problem of a multiple-reservoir system. A combination of GA and DDDP was proposed by Tospornsampan et al. [20] for irrigation reservoir operation problem. The main advantage of the hybrid approach is to save the computational resource for optimizing parameters. And also, since the good solutions obtained from GA are used as initial policy for DDDP, therefore reduces the probability of DDDP to trap in local optima. To utilize the benefits under safe flood control conditions Liu et al. [21] derived an optimal refill rule for a multi-purpose reservoir system. A hybrid multi-objective genetic algorithm (HMGA) is used in their study to integrate a progressive optimality algorithm into NSGA-II for a multi-objective reservoir.

In another popular method, simulated annealing (SA), a stochastic search technique is combined with the GA by Chiu et al. [22] for application in reservoir optimization systems. The global search was undertaken in their study by the GA and the local search by SA. In their study, the process begins with the GA while they initiate a set of random solutions and after that selection, mutation and crossover operation is carried out for a few generations. Subsequently, all of the latest solutions from the GA are handled by SA to achieve further solutions. In such manner, the algorithm used GA and SA methodology together one after another. Li and Wei [23] developed and compared the IGA-SA (improved genetic algorithm-simulated annealing) with GA, DP and SA. The IGA-SA can help to overcome premature convergence faster than the earlier GA-SA algorithm and can escape the local optima. A hybrid approach, incorporating Incremental DP (IDP) and GA was proposed by Li et al. [24]. IDP was used to provide a rough optimal solution which was used to establish a feasible subspace for generating the initial population for GA. They concluded that, since the size of the search space can be significantly reduced, the GA optimization efficiency can be considerably enhanced and the difficulty of premature convergence that is common in the practical applications of GA to reservoir operation can be partially overcome. This hybrid IDP-GA approach was applied for daily operation of a two reservoir hydropower . Authors showed that  hybrid IDP-GA approach outperformed both the univocal IDP and the classical GA.

Lerma et al. [25] demonstrated usefulness of a method based on the combined use of genetic algorithms (GA) and network flow optimization (NFO). The GA used was PIKAIA, and was linked to the SIMGES simulation model, a part of the AQUATOOL decision support system (DSS). The optimization of various operating rules was analyzed with the objective of minimizing short-term and long-term water deficits. They showed that simple operating rules produced similar results to the more sophisticated ones.

Modifications in the GA technique mainly improved the efficiency of GA for the particular problems studied and it may not necessarily be applicable to other problems therefore there is no unique method which may be applicable for all problems. So, research on hybrid GAs continues and further research scope still exists, especially for improvement in GAs or other EAs with the intension of increasing the convergence speed by finding better ways for generating initial population, reducing sensitivity analysis of operators and reducing premature convergence.

In this study an attempt is made to combine DP and GA to study a single reservoir operation problem. The methodology by combining DP and GA together is used to overcome some computational difficulties which are faced while applying DP or GA alone. GAs are robust but suffer slow convergence and need extensive sensitivity analysis to optimize different parameters to reach optimum. DP has its own limits namely, discreteness and dimensionality, however can provide global optimal solution for discrete case, but the solution is optimal for the given increment and smaller increment size will give more precise solution but will increase computational difficulties. In combined approach, DP can provide the information of the potential regions of the search space as DP provides global optimal solution for discrete case.  The initial population generated with this extra information about search space is an important help for the GA, and combined DP-GA approach may lead to faster convergence and improvement in the quality of the solutions as compared to a simple GA.

- Motivation for the proposed DP-GA approach:

When a reservoir operation problem is solved using DP the storage (state variable) is divided in suitable discrete increments to solve the model, and optimization is performed conditionally on all combinations of storages. For a large size reservoir, the size of these increments will be bigger as to use very small increment size will pose computational difficulty. Although LP can handle a large number of variables but it cannot handle non-linearity, therefore LP is useful as long as the objective function and constraints are all linear. Addition of each increment increases the computational burden of DP multiplicatively. Although discrete DP will provide global optimal solution for given sets of states and increment size, the actual optimal may lie within the discrete interval between states (Figure 1). This may not be a favoring result while the aim of the reservoir management is to efficiently and optimally use the available water, and the policy obtained based on these results may not be a good policy.

Heuristic optimization algorithms, such as, GAs are alternative tools for solving such large scale problems, however, for large size reservoirs the decision space becomes large because of a number of feasible solutions existing in the large search space, which may lead to premature convergence of these algorithms or computational difficulties as it will require significant number of generations or large population size in addition to immense sensitivity analysis of GA parameters to achieve the optimal solution.

QN*

Q1*

δ

r=N

QN

r=1

Q1

r=2

Q2

Smin=0

1δ

2δ

mδ=

Smax

m-1δ

Smin=0

1 δ

2 δ

mδ =

Smax

m-1δ

Smin=0

1 δ

2 δ

mδ =

Smax

m-1δ

Smin=0

1 δ

2 δ

mδ =

Smax

m-1δ

Smin=0

1 δ

2 δ

mδ =

Smax

m-1δ

Q2*

Figure 1 A reservoir operation problem fitted to conventional discrete DP framework

If initial search space for GA can be narrowed in a way, such that the optimal solution exists within this narrowed search space, computational requirement of GAs can be reduced upto large extent. The combined DP-GA is proposed to exploit the advantage of DP which can provide global optimal with less computational effort by using a coarse discrete increment size. Thereby, providing a potential region for the search space to be used by GA, this can further improve the solution with greater accuracy, avoiding any discreteness of decision or state variables. The DP-GA approach is used for a single reservoir operation by considering a large size reservoir of 608 106m3 capacity, as case study.

3. Methodology:

Combining the unique advantages of the DP and GA and to overcome their respective drawbacks, the DP-GA approach is devised for deriving optimal reservoir operation policy. DP with suitable discrete interval size provides a global optimal solution to a problem. This global solution derived is for the discrete values of storages chosen. However, the actual optimal solution may fall within these discrete values of storages. Therefore, a search space could be defined around this solution. With the help of these results, a search space for initial population generation for GA search could be constructed. GA is then further used to find out a better optimum with continuous state and decision variables. This also avoids any discreteness in the variables. A flow chart of combined DP-GA procedure is presented in Figure 2. After a DP-based search space is obtained, a certain number of unbiased initial individuals of GA are generated randomly within this space. The genetic operators including selection, crossover, and mutation are then used to optimize the objective function. Thus by identifying the potential region for search, a large number of needless computations of the GA can be easily avoided. Consequently, the process shall also reduce the time required to reach to the optimal solution. The prior selection of the required reasonable search space by the DP would benefit in many ways.  This would increase overall computation efficiency, in terms of deriving optimal reservoir operating policy and the corresponding required time.

3.1 Optimization with DP: Finding the search space for GA:

DP optimization is first applied to reservoir operation model to obtain an initial range for decision variables for the GA algorithm in this study. The DP model is applied to estimate the optimal monthly releases using monthly inflow to set a proper operation policy for the reservoir. The objective function is minimization of squared deviation of releases from target demands. In the DP model, the current inflow is the input to the reservoir; and the reservoir release includes the spill.  For backward process in the DP, fr(Sr) is defined as the minimum optimal return function from operation of reservoir from all the r stages to go given that reservoir storage (state) in r stages to go is Sr; gr (Sr) is the return function at r stages to go; Sr, Sr-1, Ir and EVr are the initial reservoir storage (state), the resulting reservoir storage (state), the inflow to reservoir and the evaporation losses from reservoir in r stages to go , respectively; Qr is the total reservoir release including spill, and is the decision variable for the DP model in r stages to go, respectively; Dr and Spr are the release target and the reservoir spill in r stages to go, respectively. Then the recursive equation for the DP models is:

fr Sr=minQr[grSr, Qr+ fr-1 Sr-1]  ∀r=1, …,N     (1)

The return function is:

grSr, Qr=(Qr-Dr)2       		(2)

Subject to the following constraints:

1. Reservoir continuity (state transformation) equation

Sr-1= Sr+Ir-Qr-EVr,   ∀r=1,…,N  	 (3)

2.  Spill from reservoir

Spr=Qr-Dr     ∀r=1,…,N  		 (4)

3. Other constraints related to the upper bounds are on reservoir storages (states) limited to live storage capacity (Ya) and non-negativity of state and decision variables.

Qr≥0                 ∀r=1,…,N		(5)

0≤Sr-1≤Ya   ∀r=1,…,N		(6)

The whole reservoir operation process can be represented in DP framework as shown in the Fig 1. Starting from known initial storage, possible release decisions {Q1, Q1, …,QN} at each stage should be such that the final storage belongs to the set of discrete states {Smin=0, δ, 2 δ, 3 δ,…,m δ =Smax=Ya}. Small value of ‘δ’ will increase the number of states if the reservoir is a large size reservoir, as a result computations of DP will increase. Conversely, a coarse increment size will reduce the accuracy of the optimal results, although the solution will be global optimal for a given increment. DP model solution provides a set of optimal releases {Q1*, Q2*, …,Qr*, …QN*} which is the optimal policy for given discrete increment ‘δ’.

3.2 Optimization with GA:

Real coded GA is used since floating point representation has advantages over binary, as it can represent the search space more precisely and closely to the problem [4]. The objective function of GA is the same as in DP. The constraints are also same except the limits on releases, which are defined based on the

earch space obtained from DP model solution in case of DP-GA, while for simple GA the releases are limited by target demands.

The objective function is:

Minimization of squared deviation from target demands, i.e.,

gtQt=Mint=1N(Qt-Dt)2    		(7)

where gtQt is function of release at time period t. Qt is the release for period t and Dt is target demand for time period t.

No

No

Stop

Yes

Figure 2. Flow chart of combined DP-GA approach

Define search space for GA using DP solution

{Qmin=Qt*-δ, Qmax=Qt*+δ or Dt}

DP

Optimization with

Discrete DP using increment size=δ

Start

Optimal Solution (set of optimal releases {Q1*, Q2*, …,Qt*, …QN*})

Crossover

Mutation

Termination criteria satisfied or not?

GA

Yes

Storage constraints satisfied or not?

Generate initial population randomly (gen=1) within range defined using DP, i.e.,

{Qmin=Qt*-δ, Qmax=Dt}

Calculate final storages for each gene (release) using continuity equation

Repair infeasible genes by randomly generating new ones in predefined range

Selection

gen=gen+1

The constraints for the GA model are:

The continuity equation, which is stated as:

St+1=St+It-Qt-Evt   ∀ t=1,…,N	(8)

where St, It and Qt are the storage, inflow and releases for the given reservoir at time period t and N is the time horizon for the problem under consideration. Evt is evaporation from reservoir surface during time period t, respectively.

Limits on storage impose constraints of the form

Smin≤St≤Smax    ∀ t=1,…,N 		(9)

Which ensure that the storage (St) will be within specified minimum and maximum storages.

Limits on release

Qmin≤Qt≤Qmax   ∀ t=1,…,N    		(10)

Release (Qt) should be within specified minimum and maximum range.

Most popular approach to handle constraints in GA is using penalty functions for the violating constraints and then adding them to the objective function and then solving it as unconstrained problem. The problem is finding the appropriate penalty function associated to each constraint, which requires a sensitivity analysis or prior information about the constraints to set the penalty functions. Another approach is repairing the corresponding chromosomes which create infeasibility or do not satisfy the constraints. The later approach was found to outperform penalized approached by researchers [26, 4].

Releases are the decision variables in this problem. Decision variables exist in the composition of the chromosomes of the population, [{Q11,Q21,…Qt1,…,QN1}, {Q12,Q22,…Qt2,…,QN2},… , {Q1NP,Q2NP,…QtNP,…,QNNP}]. Since initial population is generated within the limits specified for the decision variables, so the release constraints are readily satisfied (Eq. 10). In case of simple GA decision variables are generated within the entire range {Qmin=0, Qmax=Dt }. The search space for DP-GA is defined as {Qmin=Qt*-δ, Qmax=Qt*+δ }, that is in δ vicinity of the optimal release made by DP (Figure 1). To further refine this range the releases were generated within the range {Qmin=Qt*-δ, Qmax=Dt}, i.e., the upper bounds on releases are kept as the target demands. Continuity equation is also satisfied beforehand, since the storages are computed using the continuity equation itself given in Eqn. (8).

Limits on storages are the constraints which need to be checked for their feasibility during GA computations. Assuming given initial storage at the beginning of the operation period for each chromosome, final storages are computed for each gene (release) using the continuity equation. Since, releases are generated randomly within specified limits, the corresponding storages may or may not lie within the range specified for them. During GA computations, if a storage value is less than the predefined lower limit, then the corresponding gene will be repaired and a new gene will be randomly generated in the search space, and will be checked for feasibility, if the corresponding storage found infeasible again then this cycle will be repeated until a new gene is found which satisfy the lower limit on the storage. In case, the storage value exceeds the upper limit, the final storage will be equal the storage capacity of the reservoir and rest will be defined as spill (Spt) from the reservoir. The same will be repeated for all genes of each chromosome in the population to calculate final storages associated with each release until a feasible initial population is found. This population will further go through selection, crossover and mutation process to find new generation of the parent population. The new population will now be checked for feasibility and will be refreshed if needed before it goes through GA operators. This process will be repeated for predefined maximum number of generations. The operators used in GA process are discussed below:

3.1.1 Selection: Most common among the selection methods, that is, Roulette wheel selection method is used in this study. Goldberg [2] reported that the simplest selection method was the roulette wheel method. The basic implementation of a roulette wheel selection method assigns each chromosome a “slice” of the wheel, with the size of the slice proportional to the fitness value of the chromosome. In other words, the fitter a member is the bigger the slice of the wheel it gets. To select a chromosome for selection, the roulette wheel is “spun,” and the chromosome corresponding to the slice at the point where the wheel stops on is grabbed as the one to survive in the offspring generation.

The main computational steps for the roulette wheel selection method are as follows:

1. Fitness of each chromosome, fti, and their sum i=1NPfti are calculated, where the population size is NP.

2. A real random number, rand(), within the range [0, 1] is generated for each chromosome and s is set to be equal to the multiplication of this random number by the sum of the fitness values, i.e., s= rand()×i=1NPfti.

3. The kth chromosome is selected for the next generation, determining the value of k such that s is less than or equal to the cumulative sum of finesses upto kth chromosome, that is, s≤i=1kfti.

4. Steps 2 and 3 are repeated until the number of selected chromosomes becomes equal to the population size, NP.

3.1.2 Crossover:  Crossover operator can significantly influence the performance of real coded genetic algorithms as the exploration and exploitation of the search space is governed by crossover operator being applied. Generally, BLX-α crossover gives good results [11, 4].

BLX-α Crossover

Two offspring y1=(y11,y21, …, yn1) and y2=(y12,y22, …, yn2) are generated.

where,  yik is a randomly uniformly chosen number from the interval Xmin- Iα, Xmax+Iα

and Xmax, Xmin and I are defined as below

Xmax=maxxi1, xi2,			(11)

Xmin=minxi1, xi2, and			(12)

I=Xmax-Xmin				(13)

Higher value of α results in a better solution. As α is increased, the exploration level increases, since the relaxed exploitation zones spread over exploration zones, thereby, increasing the diversity levels in the population [27].

3.1.3 Mutation: Random mutation is performed using a mutation probability which is considered as inverse of the population size for each case in this study. In random mutation, the mutated gene is replaced with a new randomly generated gene within the specified search space.

- Study area:

The Mula project envisages a major multipurpose reservoir across the river Mula, a subtributary of the river Pravara in the Godavari river basin in the State of Maharashtra in India. It was initially constructed to serve the irrigation water needs of the system only. Subsequently a provision for water supply to Ahmednagar city,

industries and villages was made. The total annual water requirements are 748106 m3 out of which the annual irrigation water demand is 674106 m3 and annual water supply requirements are 74106 m3. A data series of 18-years-monthly inflow to the reservoir is available. The mean annual and the 75% water year dependable inflows to the reservoir are 825 and 688 106 m3, respectively. Based on the historical data set, the estimates of the mean and the 75% water year dependable inflows to the reservoir were obtained from the flow duration curve analysis. The gross reservoir capacity is 736106 m3 active and dead storage capacities are 608 and 127106 m3, respectively. There is no provision for the flood storage.

- Results and Discussion:

The operation of the single reservoir Mula was taken as case study to evaluate the performance of DP-GA approach. The operation was done for an average year using the 18 years historical data. The average monthly average flow and average monthly target demands are shown in Figure 3. The DP-GA optimization results were compared with DP and simple GA. DP model is first solved to find an optimal policy with discrete state increment 8 106 m3.

The release policy using DP for an average year is shown in Figure 4. Variations in the releases from target demands can be seen from this figure. The value of objective function is 70.31, which is the minimum squared deviation from target demands. For an average flow there should not be any deficit as sufficient water is available for release, and this variation in releases is mainly due to the increment used in DP. DP underestimates or overestimates the decisions because of increment size used to discrete the storages or state variables. For large size reservoir increment size will be taken larger to reduce computational difficulty and the variation in releases will be more than in this case. Therefore the results obtained are optimal for given increment but are not actually optimal as these results needs to be further refined. This can be done by using another approach which can search for the actual optimal within this space already searched by DP. In this study such an attempt is made by using GA to refine the results obtained by DP. GA searches for the optimal in the space bounded by upper limit as Qt*+δ and lower limits as Qt*-δ. The Figure 4 shows the search space facilitated by DP for DP-GA approach.

This space can be decreased or increased depending on the problem. In present study the space was further refined as {Qt*-δ ,Dt}, as release should not be bigger than the target demand.

A computer code has been developed in FORTRAN. The code was first developed for simple GA and tested for some standard problems. It was further modified for reservoir operation problem to be solved using simple GA or DP-GA. No intense sensitivity analysis was done on GA parameters. Crossover and mutation probabilities were fixed after a few numbers of trials. For the BLX-α crossover operator the value of α was fixed at 0.5. For fixing the probability of crossover, a range 0.2, 0.4, 0.5, 0.6, 0.8, 0.9 of probabilities was tested and it was seen that better results were obtained for larger value of crossover probability, as suggested by many researchers too. Figure 5 shows the effect of crossover

Figure 4. Search space facilitated by DP for DPGA

Figure 3. Average Monthly inflow and demand at Mula reservoir

Figure 5. Convergence with different Crossover rates for population size=50

Figure 6. Convergence with Mutation rate variation for population size=50

Table 1: Performance evaluation of Simple GA and DPGA

| Population size | Max. no. of generations | Generation no. when optimum reached |  | Value of Objective function |  | Computation time (Seconds) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GA | DPGA | GA | DPGA | GA | DPGA |
| 10 | 500 | 400 | 301 | 162.156 | 30.473 | 2- 3 | 1 - 2 |
|  | 1000 | 990 | 997 | 86.742 | 4.934 | 2 - 3 | 1 - 2 |
|  | 5000 | 3753 | 1746 | 8.873 | 0.449 | 8 - 9 | 8 |
| 20 | 500 | 500 | 473 | 24.142 | 0.969 | 3 - 4 | 2 - 3 |
|  | 1000 | 999 | 666 | 4.033 | 0.120 | 5 | 5 |
|  | 5000 | 4217 | 2245 | 1.020 | 0.065 | 25 | 23-24 |
| 50 | 500 | 500 | 419 | 2.095 | 0.120 | 13 | 12 |
|  | 1000 | 930 | 999 | 0.224 | 0.036 | 24 | 22-23 |
|  | 5000 | 4208 | 1215 | 0.208 | 0.017 | 115 | 102 |
| 100 | 500 | 498 | 497 | 2.095 | 0.043 | 45 | 41-42 |
|  | 1000 | 980 | 518 | 0.241 | 0.009 | 87 | 85 |
|  | 5000 | 4179 | 2099 | 0.017 | 0.008 | 345 | 343 |
| 150 | 500 | 487 | 365 | 0.184 | 0.041 | 83 | 86 |
|  | 1000 | 876 | 888 | 0.018 | 0.007 | 192 | 176 |
|  | 5000 | 3155 | 2387 | 0.005 | 0.005 | 950 | 956 |

probability on performance of GA for population size 50. Finally the probability of crossover was set to be 0.8. Different mutation probabilities were tried for population size 50, see Figure 6. The mutation probability was found best as inverse of the population size. However, for present study it was taken as 0.01 in each case. Besides this no further sensitivity analysis was made on type of crossover or mutation operators. Most commonly used selection crossover and mutation methods were used.

Population sizes and number of generations were varied to see their effect on optimization approach since large population size and large number of functions evaluations increases the computational requirement of GA and make convergence slower. The limits on decision variables which was 0 to Dt for simple GA and {Qt*-δ ,Dt}, for DP-GA while all other parameters were kept same for simple GA and DP-GA in order to make a comparison.

Results were obtained for population sizes 10, 20, 50, 100 and 150 using both DP-GA and simple GA. For each population size results were obtained for 500, 1000 and 5000 generations. Table 1 presents the value of objective function, number of generation where the optimum was found and computation time taken in each case. Convergence of DP-GA and simple GA is shown in Figs.7a-7e for different population sizes for 5000 functions evaluations. These figures clearly explain the difference in the performance of the two approaches. DP-GA starts to converge towards optimum faster than simple GA. Figure 8 shows that the optimal results for 100 population size for 5000 generations using simple GA are equivalent to the optimal results for population size 20 for DP-GA for same number of generations. The computation time for each case is 24 secs and 345 secs, respectively. This shows that DP-GA reaches to the same optimal more than 10 times faster. Similarly it can be seen that similar results can be obtained with population size 50 in 1000 generations or with a population size 100 in 500 generations (see Table 1 and Figure 8). In each case it can be seen that DP-GA can reach to optimal approximately 10 times faster as compare to simple GA, either with small population size and larger number of generations or with bigger population size and less number of generations (Figure 9). In every case the computational requirement can be reduced approximately 10 times as compared to simple GA.  DP-GA with less computational efforts gives better results as compared to simple GA or DP as shown in Figure 9.

The optimal release policies obtained for the two cases from Figure 9, that is, case i. for population size 20, 5000 generations and case ii. Population size 100, 500 generation using both simple GA and DP-GA are presented in Figure 10 and 11, respectively. The monthly operation policies derived using both the DP-GA and the simple GA are very similar and better as compared to DP. But, the computational requirement is considerably lessened using DP-GA as compared to the GA.

- Conclusions

The results of the combined DP-GA approach used to derive optimal reservoir operation policy of the Mula reservoir seems to be quite promising. The approach helped in reducing the computational requirements of simple GA. The convergence of the DP-GA is approximately 10 times faster than the simple GA. Similar results were reported in various applications, who used IDP and GA together [24]. The use of simple GA gives results similar to the DP-GA but at the cost of computational requirement. Therefore, it can be said that the DP-GA outperforms the simple GA in giving more precise results with less computational requirement. This could be achieved in the GA by using small population sizes or by using less numbers of generations, since convergence becomes quite slower if population size is bigger. Because, the DP-GA search approach provides a narrowed search space for deriving an optimal solution. This approach is more useful for large size reservoirs where the decision space for the variables under consideration is large in size. In such cases a coarse increment size is to be used to discrete state variables for DP, while the search space becomes very large for GAs for finding optimum. The proposed method shall be applicable to the problems which can be fitted into DP framework. The operation policies derived using both the DP-GA and the simple GA are similar. But, the computational requirement is considerably lessened using DP-GA as compared to the GA.

- (b)

(c)                        					(d)

(e)

Figure 7. (a)-(e) Convergence of simple GA and DPGA with increase in population size

Figure 9. Comparison of DP, simple GA and DPGA performances

Figure 8. Comparison of GA and DPGA for different	population sizes

20

10

Figure 11. Release policy for DP, simple GA and DPGA for case ii

Figure 10. Release policy for DP, simple GA and DPGA for case i

- Acknowledgements:

Financial support provided by MHRD to work at IIT Roorkee is gratefully acknowledged by the first author.

- References:

- Holland, J. H., Adaptation in Natural and Artificial Systems, University of Michigan Press, Ann Arbor, 1975.

- Goldberg,D. E., Genetic Algorithms in Search, Optimization, and Machine Learning, Addison-Wesley, Reading, MA, 1989.

- Davis, L., Handbook of Genetic Algorithms, Van Nostrand Reinhold, New York, 1991.

- Michalewicz, Z., Genetic algorithms + data structures = evolution programs, 3rd rev. and extended ed. Springer, New York, 1996.

- Mitchell, M., An Introduction to Genetic Algorithms, MIT Press, Cambridge, MA, 1999.

- Deb, K., Multi-objective optimization using evolutionary algorithms, John Willey & Sons (Asia), Singapore, 2003.

- Haupt, R. L. and Haupt, S. E., Practical genetic algorithms, John Wiley & Sons, Inc., Hoboken, New Jersey, 2004.

- Ganji, A., Karamouz, M. and Khalili, D., “Development of stochastic conflict resolution models for reservoir operation. II. The value of players’ information availability and cooperative behavior”, Advances in Water Resources, Vol. 30, pp. 528–542, 2007.

- Karamouz, M., Mojahedi, A. and  Ahmadi, A., “Economic assessment of operational policies of inter-basin water transfer”, Water Resources Research Vol. 3, No. 2, pp. 86–101, 2007.

- Chen, L., Chang, F.J., “Applying a real-coded multi-population genetic algorithm to multi-reservoir operation”, Hydrological Processes ,Vol. 21, No. 5, pp. 688-698, 2007.

- Hınçal, O,  Altan-Sakarya, A. B. and  Ger, A. M., “Optimization of multireservoir systems by Genetic Algorithm”, Water Resources Management Vo. 25, pp. 1465–1487, 2011.

- Wensheng, D., Xinzhong, Z. and Liu, “Adaptive Immune genetic algorithm for optimization reservoir dispatching”, In: Int. Conf. on Computer and Communication Technologies in Agri. Eng. (CCTAE), pp 41–44, IEEE, 2010.

- Yun, R., Singh, V.P. and Dong, Z., "Long-term stochastic reservoir operation using noisy genetic algorithm", Water Resources Management, Vol. 24, pp. 3159–3172, 2010.

- Cheng, C.-T., Wang, W.-C., Xu, D.-M. and Chau, K.W. , "Optimizing hydropower reservoir operation using hybrid genetic algorithm and chaos", Water Resources Management, Vol. 22, pp. 895–909, 2008.

- Hossain, M.S. and El-shafie, A., "Intelligent Systems in Optimizing Reservoir Operation Policy: A Review", Water Resour Manage, Vol. 27, pp. 3387–3407, 2013.

- Rani, D. and Moreira, M. M., “Simulation-optimization modeling: A survey and potential application in reservoir systems operation”, Water Resources Management, Vol. 24, No. 6, pp. 1107-1138, 2010.

- Cai, X.M., McKinney, D.C. and Lasdon, L.S., “Solving nonlinear water management models using a combined genetic algorithm and linear programming approach”, Advances in Water Resources,  Vol. 24, No. 6, pp. 667-676, 2001.

- Reis, L., Bessler, F., Walters, G. and Savic, D., “Water supply reservoir operation by combined genetic algorithm-linear programming (GA-LP) approach”, Water Resources Management , Vol. 20, No. 2, pp. 227-255, 2006.

- Huang, W.C., Yuan, L.C. and Lee, C.M., “Linking genetic algorithms with stochastic dynamic programming to the long-term operation of a multireservoir system”, Water Resources Research, Vol. 38, No. 12, pp. 40-1-40-9, 2002.

- Tospornsampan, J., Kita, I., Ishii, M. and Kitamura, Y., “Optimization of a multiple reservoir system operation using a combination of genetic algorithm and discrete differential dynamic programming: a case study in Mae Klong system, Thailand”, Paddy and Water Environment,Vol. 3, pp. 29–38, 2005.

- Liu X. Guo S., Liu P., Chen L. and Li X., “Deriving Optimal Refill Rules for Multi-Purpose Reservoir Operation”, Water Resour Manage, Vol. 25, pp. 431–448, 2011.

- Chiu, Y-C., Chang, L-C. and Chang F-J., “Using a hybrid genetic algorithm-simulated annealing algorithm for fuzzy programming of reservoir operation”, Hydrol Process, Vol. 21, pp. 3162–3172, 2007.

- Li, X-G. and Wei, X., An Improved Genetic Algorithm-Simulated Annealing Hybrid Algorithm for the Optimization of Multiple Reservoirs. Water Resour. Manage., Vol. 22, pp. 1031–1049, 2008.

- Li, F-F.,  Wei, J-H.,  Fu, X-D. and  Wan, X-Y., “An Effective Approach to Long-Term Optimal Operation of Large-Scale Reservoir Systems: Case Study of the Three Gorges System”, Water Resour Manage, Vol. 26, No. 14, pp. 4073-4090, 2012.

- Lerma N., Paredes-Arquiola J., Andreu J. and Solera A., “Development of operating rules for a complex multi-reservoir system by coupling genetic algorithms and network optimization”, Hydrological Sciences Journal,Vol. 58. No. 4, pp. 797-812, 2013.

- Alghazi, A., Elazouni, A.  and Selim, S., “Improved Genetic Algorithm for Finance-Based Scheduling”, Journal of Computing in Civil Engineering, ASCE, Vol. 27, No. 4, pp. 379-394, 2013.

- Herrera, F., Lozano, M. and Verdegay, J.L., “Tackling realcoded genetic algorithms: Operators and tools for behavioural analysis”, Artificial Intelligence Review, Vol. 12, pp. 265–319, 1998.