IMPROVEMENTS IN CANALS OF GUDHA IRRIGATION PROJECT

Y.C.AGRAWAL

Abstract: Gudha project was selected for Action Research Programme under the Irrigation Management & Training Project sponsored by WAPCOS and USAID. The objective of Action Research Studies is to identify deficiencies, suggest cost & time effective interventions for optimal utilisation of irrigation water for crop production including monitoring & evaluation of interventions. Two minors were selected for Action Research Work, work on which was started in May 1985. Most of the work was completed by middle of year 1990. The improvements benefited the farmers. Myself worked as Research Officer (Irrigation) in this programme since September 1986 to April 1989. This paper is based on the work done by me.

1.0   INTRODUCTION

Gudha Project is situated in Bundi district of Rajasthan (India). It is in operation since 1958-59.

The catchment area of Gudha Dam reservoir is 753.69 sq.kms (291 sq. miles). The gross storage capacity of dam at FTL is 95.56 million Cum (3375 M.cft). Dead storage is 1.98 million Cum (70 M.cft). The culturable commanded area of the project is 10.860 ha as mentioned in Modernisation Project Report. The irrigation intensity has been envisaged as 98.50% ,85% in Rabi &  13.50% in Kharif. The maximum irrigation so far achieved is 8675.84 ha.(21,438 Acres). Thus 79.92% irrigation intensity has been achieved so far, out of which 15.75% (1709.83 ha or 4225 Acres) is in Kharif and 61.44% (6670.17 ha or 16482 Acres) in Rabi. The duty of water was envisaged as 114.33 ha per M.Cum(8 Acres per M.cft), but maximum duty of 127.77 ha per M.Cum (8.96 Acres/M.cft) could be achieved in the year 1985-86 since raising the Full Tank Level by 1.07 m(3.50 ft) in the year 1983.

In the Modernisation Project Report duty of water at outlet head has been mentioned as 1286.26 ha per cumec (1 cusec for 90 Acres). In the command area of this project, there are about 3000 wells, which are used for irrigation also. With the help of these wells, sugarcane is also produced in this area. Since September 1986, water levels in about 100 wells were recorded, scattered in whole of the Gudha command. These observations show lowering of water level in the wells. However as per water levels, recorded in past irrigation period in January 1989, the water level was within root zone (0-1.5 m) of the plants in some scattered patches. Such patches are more in left command as compared to right  command.

The Irrigation Management under Action Research Programme consists of intensive attack on real problems of irrigation system by detailed and comprehensive interventions based on the new concepts. The status report of Gudha Project was prepared in September 1985 and was submitted to the concerned authorities. Two minors were selected for Action Research Project, one in right command and other in left command.(Figure 1)

(i)Minor No.3 of Right Main Canal

(ii)Minor No.2 of Left Main Canal

2.0  MINOR NO.3 OF RIGHT MAIN CANAL

This minor off takes from left bank of Right main canal at its chain 389. This minor irrigates the lands of two villages Sathur and Barodia. Minor was having one head regulator containing two rows of 0.30 m(one ft) diameter pipes. The design discharge of minor was  only 22.65 litres per second (8 cusecs). The existing length of minor was 2.28 Km(7500 ft). The irrigation done in 1985-86 was 27.52 ha (68 Acres) in Kharif and 198.70 ha (491 Acres) in Rabi, total 226.22 ha (559 Acres).

The culturable commanded area of the minor was assessed, which was found as 536.22 ha(1325 Acres). As such minor was redesigned for a water allowance of 90 Acres per cusec at outlet head, with intensity of irrigation as 98.5%. As per revised draw off statement, the design discharge is 433 litres /sec (15.29 cusec). The proposed bed width, full supply depth and bed slope at head are 1.7 m(5.6 ft),0.50 m(1.65 ft) and 1 in 2500 respectively for masonry lined rectangular section. To cover whole of  the CCA , the Minor has been extended by 1.28 Km(4100 ft) making the total length of minor as 3.56 Km (11700 ft).

At Km 0.88 (chain 29) , a sub minor takes off from left bank of the minor. CCA and design discharge of sub minor are 121.80 ha(301 Acres) and 101 litres/sec(3.57 cusec). The length of this new sub minor is 1.74 Km (5700 ft) . The bed width, full supply depth and bed slope have been kept as 0.91 m(3 feet), 0.34 m(1.1 feet) and 1 in 4000 for a trapezoidal earthen section with side slope 1:1.(Figure 2 & 3)

2.1  INSUFFICIENT CONTROL STRUCTURES AND BRIDGES

Except one head regulator, there was no masonry structure across the minor . As such two falls at Ch.30 & 91, and three fall cum VRB's at Ch 17,55 & 83 have been constructed . One 0.45 m (1.5 ft) diameter pipe type VRB has been constructed at chain 117 (tail) . Two drainage crossings have been constructed at Ch.98 and 116.50. Also nine  APM's have been constructed.      On Sub Minor two VRB at chain 2 and 55 have been completed. Fall cum VRB at Ch 10.50 and Fall at Ch 30 have been constructed.

A new head regulator of minor has been constructed with 0.91 m(3ft) wide and 0.61 m(2 ft) deep opening. One head regulator of sub minor with 0.61 m (2 ft.) wide opening has also been constructed.

2.2  LACK OF MEASURING DEVICE AT HEAD AND TAIL

It is proposed to utilise the first fall at ch.17 as a measuring structure. At tail, a gauge pillar will be provided to determine the discharge at tail.

2.3  WASTAGE OF WATER DUE TO SEEPAGE/EXCESS LOSSES

2.3.1 Seepage loss studies were done by Ponding method in four 	 reaches as shown in Table 1.

2.3.2. The soil strata of the bed of minor was got tested for 	  grain size distribution of the soil. The results have been         given in Table 2.

2.3.3. The minor has been lined in following reaches to check the 	excessive seepage losses and erosion of minor bed & sides below falls.

(1) Chain 0 to chain 19

(2) Chain 25 to chain 32

(3) Chain 55 to chain 57

(4) Chain 65 to chain 91

(5) Chain 91 to chain 93.

Lining has been proposed in the following reaches to check the excessive seepage losses.

(1) Chain 93 to chain 117

(2) Chain 0 to Chain 10.50 of sub	minor.

2.4  CONSTRUCTION OF WATER COURSES

For the micro net work planning, for construction of water courses, topographical survey of the command area in a grid of 15 m x 15 m with 10 cm contour interval is being done by OFD wing of CAD Chambal Kota. Micro net work planning will be done after receiving contour map of the command area.

2.5  EVALUATION OF INTERVENTIONS AND FURTHER IMPROVEMENTS

During the year 1987-88, the minor could irrigate only 259.41 ha(641 Acres), 27.52 ha(68 Acres) in Kharif and 231.89 ha(573 Acres) in Rabi.

Also in Feb. 1988 , the discharge of the minor was observed by surface float method. The actual discharge at FSD was 7.50 cusecs against a design discharge of 15.29 cusecs. As such the design of  minor and its falls were checked.

(1)	It was observed that rugosity coefficient for design of masonry lining from chain 12 to 17 was considered as 0.018, which should have been considered as 0.025.

(2)	The crest level of falls are generally 10 cm to 15 cm higher than the designed levels. As such these should be lowered.	The crest has been lowered  in November, 1988.

(3)	The water way width of the falls should be equal to the water surface width at FSD in the minor downstream of fall.

As such water way width of crests of falls is to be increased by 0.45 m (1.,5 ft) at Ch.17. The width is to be increased like wise at other falls.

(4)	The effect of removing the excessive height of crest of falls, on the carrying capacity of minor was observed in 1988-89. It was seen that at Full Supply Depth, the minor was able to carry 10 cusec discharge, instead of 7.50 cusecs carried in Feb. 1988.

(5)	Masonry lining has also been done from chain 0 to 19 of the minor. It will further help in improving the carrying capacity of minor from Ch.0 to Ch.17.

(6)	At well type fall at chain 91, there were no upstream wings. This has caused breaching of banks of minor, scouring in bed & sides upstream of fall, and silting of bed downstream of fall.

(7)	Looking to the soil strata containing 64% sand and presence of salts, it is proposed to provide masonry lining from chain 85 to chain 117(tail) to check excessive seepage losses and damage to the banks of minor.

(8)	The cultivators have not constructed water courses in the increased command area of the minor . Therefore only small increase in area irrigated has been observed.

3.0  MINOR NO.2 OF LEFT MAIN CANAL

It off takes from LMC at its chain 153 (Km 4.663)from right bank, near Raghunathpura village. It has a 0.61 m(2ft) diameter pipe type gated head regulator. Existing length of minor is 5670 m (18,600 ft) . The minor does not have any other control structure. The minor irrigates the cultivated lands of villages Reghunathpura, Mangli Kalan, Mangli Khurd, Bichari and Chatarganj. The existing CCA of minor is 496.96 ha. (1228 Acres) .

The minor irrigated 401.10 ha .(991 Acres) i.e. 80% of CCA in the year 1987-88; 320.92 ha(793 Acres) in Rabi and 80.13 ha (198 Acres) in Kharif.  The main crops grown in the area are wheat (247.1 ha i.e. 477 Acres),  Pea (83.77 ha i.e. 207 Acres) and  sugarcane (80.13 ha i.e. 198 Acres).

Old design discharge of minor was 155 litres/sec (5.48 cusecs). Masonry structures like VRB falls etc. were designed to carry this design discharge. There are 16 VRB's, falls etc. across this minor. The falls were in very bad condition. Minor was no where lined. The number of authorised outlets was 28, but 42 pipe type outlets were counted which were of six inch diameter. There was a Replogle flume at RD 45 M (150 ft), crest of which was damaged by the cultivators.

The old length of minor was 5.67 Km.(18600 ft). From head regulator to its crossing with National Highway, the length of minor is 5.03 Km. (16,500 ft) and below National highway the existing length was only 0.64 Km.(2100 ft).

The old culturable commanded area of the minor was assessed as 488.76 ha (1207.72 Acres). This includes 189.36 ha (467.90 Acres) below National highway (Km.5.03 ).(Figure 4 & 5)

3.1	ALIGNMENT OF MINOR

3.1.1 HEAD REGULATOR TO ITS CROSSING WITH NATIONAL HIGHWAY

The present alignment of minor from Km 2.90 (9514 ft) to Km 5.03 (16502 ft) is very near to the river and on right side of the command area of this reach. This resulted in long length of minor, excessive seepage losses in the minor and long length of water courses.

As such a new straight alignment was proposed on left side 	of existing road. But some difficulties were faced.

(i) In the middle reach, the soil strata was Jhinkra(stratified soft rock). In this reach, the seepage losses were expected to be high and due to this reason, lining would have been essential.

(ii) In the head and tail reaches ,the minor was in filling 	by about 0.90 to 1.80 m(3 to 6 ft) . In these reaches, 	cultivators opposed the construction of minor in their 	fields. Also due to heavy filling there would have been 	necessity of lining the minor.

Looking to the above two reasons, proposal of change in alignment of minor in this reach was dropped.

3.1.2  NATIONAL HIGHWAY CROSSING TO TAIL (OLD) END

Below National Highway Km 5.03  (16502 ft), the minor turns to the left for its remaining length of 0.64 Km(2100 ft). But it was unable to serve the present CCA of 189.86 ha. (467.90 Acres). As such the alignment of the minor below National Highway has been changed and minor will adopt the alignment of existing water course, after considering the three proposals mentioned below.

(i) First proposal was to construct the minor along existing water course in a length of about 518 m(1700 ft) and then along existing cart track which was in the middle of the command area. But it was seen that beyond one Km length, there were low lying fields. As such minor was to go in filling.

(ii) Second proposal was to construct minor along existing water course 0.518 Km ( 1700') long. Then construct it along existing cart track (600 ft in length) and then turning right angles along a existing cart track and then turning left at right  angles to end at the existing lift water course.

Although as far as possible, the alignment was in the middle of command area, but after 518 m (1700 ft), it was to go in filling. Also cultivators were opposed to this alignment on the grounds that  (a) it will spoil their cart track due to seepage from minor, (b) minor will be constructed in their fields and (c)  minor will also cause stagnation of water in their fields.

(iii) The third proposal was to construct the minor by improving the existing water course in a length of 1740 m. This alignment is not in filling. This alignment is on right side of the command area. At  some distance minor will run very near to the Mej river which is 9-12 m (30'-40') deep below ground level.

The cultivators also wanted to get the minor constructed along this existing water course, because

(a)	They will loose no land for construction of minor.

(b)	They have existing system of branch water courses off taking from this main water course, to irrigate their lands.

(c)	Their cart track will not be affected  due to seepage from minor/new water courses.

(d)	Their fields are sloping away from this main water course.

Considering the above facts, the alignment  of minor beyond National Highway was finalised along the existing main water course.

3.2  WATER ALLOWANCE

In Gudha Modernisation Project report, the water allowance has been taken as 1.29 ha per litres/sec (90 Acres per cusec)at outlet head. The action research unit decided to keep the same water allowance.

3.3  COMMAND AREA OF MINOR ----- CONTOUR SURVEY

The ground levels were taken along lines 152.4 m (500 ft ) apart and perpendicular to the minor. Levels at every 30.50 m(100 ft) were taken along such lines. On the basis of this survey, contours were drawn.

3.3.1. Shifting of chak No.1 left side & right side to L.M.C.:

It was observed that field levels of left side and right side Chak No.1 are too high to be irrigated by the Minor in a proper way. As such fresh levels were taken in these two chaks in a grid of 30.5 x 30.5 m (100 ft  x 100 ft) and fresh contours were drawn. These contours confirmed the earlier opinion that the two chaks could not be irrigated by the minor in a proper way. At present cultivators used to put obstructions in the minor at existing first fall at RD 465 m  to raise the FSL in the minor as high as possible to irrigate their fields and thus obstructing flow of design discharge in the minor.

As such these two chaks have been shifted to the parent canal i.e. Left Main Canal. Right side chak No.1 (CCA 9.66 ha or 23.888 Acres) could be irrigated by an existing out let at RD 14360 feet of the main canal. For left side chak No.1 (CCA 14.35 ha or 35.46 Acres) a new outlet has been provided in LMC . Two lined link water courses will be constructed from these outlets to provide water for  these two chaks.

3.4  REVISED L-SECTION OF MINOR

It was decided to keep the minimum outlet discharge as 14.15 litres/sec.(0.5 cusecs) and accordingly minimum chalk size has been kept 18.21 ha (45 Acres). with the help of contour sheet, the command area has been divided into 19 new chalks . The conveyance losses have been considered as 2.44 cumec/M.Sq.m.(8 cusecs per million square feet) of wetted perimeter of minor. As per draw off statement, the design discharge of the minor at head has been kept as 5.67 litres/sec (14.03 cusecs) for a CCA of 464.75 ha (1148 Acres) and water allowance of one cusec for 90 Acres as compared to earlier design discharge of 5.48 cusecs.

The command area is having rock out - crop at many places. The fields are sloping away from these out crops on all sides. Also cultivators provide temporary obstructions in the minor at many places to raise the existing FSL in the minor by about 0.30 m (one foot).

The two chaks in the head reach have been transferred to the parent  canal i.e. L M C. As such, in the head reach, the bed of the minor has been lowered by about 0.60 m (2 ft) to avoid any unauthorised withdrawal of water from minor for these two old chaks, to keep the minor in cutting and avoid existing fall at RD 120 m (395 feet).

In the remaining length of minor existing FSL has been raised in such a way that minor may irrigate the commanded area by flow irrigation easily except a few higher patches of few Acres. These higher patches could be irrigated by lifting water from water courses, as too much higher FSL would have caused seepage and salinity problems in the fields adjoining to the minor. Generally the bed slope was proposed as 1 in 2000 and V/Vo was proposed more than one. The bed width and FSD were proposed  as 1.07 m (3.50 ft) and 0.53 m (1.75 ft).

3.5  REMODELLING OF MASONRY STRUCTURES

In the revised design as per intervention, design discharge of minor has increased and full supply level has been raised. This has caused the necessity of remodelling the existing VRB's and falls. This was taken up in hand. Also two new falls will be provided at RD 1140 m (3740 ft) and RD 3720 m (12205 ft) with a drop of 0.30 and 0.32 m. Also the existing one ft diameter pipe DRB across the National Highway should be removed and a new slab type VRB should be constructed for increased design discharge of 0.156 cumec (5.51 cusecs). The minor should be extended below National Highway along its existing water course. New falls and VRB's will be constructed across the minor in this portion.

3.6  WATER LOGGING/SALINITY PROBLEM

There is problem of stagnation of water, salinity and water logging in the head reach of minor. Problem will get solved after implementation of proposed improvements in the minor.

3.7  MEASURING STRUCTURES AT HEAD AND TAIL

The first fall at RD 465 m (1525 ft) with a drop of 0.75 m will be used as a Meter fall. After completion of extension of minor, a gauge pillar will be provided at tail to determine the discharge.

3.8  WASTAGE OF WATER DUE TO SEEPAGE/EXCESS LOSSES

3.8.1. The seepage losses in the minor were  measured by Ponding 	method at many places, in head, middle and tail reaches. These are mentioned in Table 3.

3.8.2. The soil strata of  the bed of minor was got tested for grain size distribution of the soil. The results have been shown in Table 4.

There is good service road along the minor. Also there is a metalled road from RD 1370 m to RD 5030 m parallel and near to  the minor.

3.8.3. Considering the above seepage losses, it has been decided 	 to provide lining in the minor in its following reaches.

(1)	RD 0 to 300 m

(2)	RD 15 30 m to 1620 m

(3)	RD 3600 m to 3825 m

(4)	RD 4050 m to 4760 m (VRB)

(5)	RD 5070 m (fall) to RD 5259 m

3.9  UNAUTHORISED OUTLETS/OVER SIZED OUTLETS

Against 28 authorised outlets there are 42 outlets, all but one of 6" diameter. Now only 19 outlets have been proposed. As such all existing outlets will be removed and properly designed APM's will be provided at these 19 places.

3.10  NON AVAILABILITY OF WATER IN TAIL REACHES

Due to construction of fall at RD 465 m and proper raising of banks in head reaches, the availability of water has increased below National Highway in a CCA of 189.86 ha (467.90 Acres), as per the gauges recorded and personal enquiry from cultivators below National Highway.

4.0  WATER DISTRIBUTION IN Gudha COMMAND

The water scheduling in whole of the Gudha command is decided by Water Distribution committee comprising of District  Collector, Officers of Irrigation and Agriculture departments, public representatives and cultivators. As per actual requirements and winter rainfall, some deviations are made by the Executive Engineer Irrigation Division Bundi in consultation with all related people.

However it is estimated that after completion of works as per Action Research Programme findings and as envisaged in Modernisation Project Report, the water management in the Gudha command will improve.

Conclusion:

1. L-section of a canal should be finalised on the basis of micro     network survey and chak boundaries.

2. Pipes should not be used in any masonry structure across  a     canal i.e. in head regulator, road bridges,   siphons, etc.     Only slab type openings should be provided across the canal     to avoid head loss in blockage of pipes by miscreants.

3. Lining should be provided in the canal in reaches of excessive     seepage losses and dispersive soil.

4. Conjunctive use of canal and ground water should be encouraged     by creating canal water scarcity and giving incentive   for     using ground water.

5. The water courses should be lined in a length of at least 30     m. in the head reach below the outlet and   other   filling     reaches to avoid loss of head/water.

6. The rugosity coefficient in canal design should be adopted      after carefully considering all factors and performance of      canals in the adjoining areas.

7. Fluming of vertical falls should never be done. The length of     crest of fall perpendicular to the flow direction should be     equal to downstream water surface width at full supply depth.

8. Well type falls should be properly designed and proper energy     dissipation arrangements should be provided. Upstream wings     in a suitable length should be provided.

9. Cultivators should be educated to use   water   efficiently     adopting modern methods of irrigation and to prevent   wastage     of water.

TABLE 1.Seepage loss study in Minor No.3 by ponding method

| S.No. (1) | Reach (2) | Seepage losses in cumec per M.sqm. (cusecs per M.sft) of wetted perimeter (3) | Type of strata of bed and sides (4) |
| --- | --- | --- | --- |
| 1 | chain 7 to 10 (FSL below NSL) | 3.96 (13) | clay with kankar (5%) |
| 2 | chain 39 to 40 (FSL below NSL) | 2.59 (8.50) | clay |
| 3 | chain 59 to 60 (FSL below NSL) | 4.88 (16) | soft rock/murrum with kankar (25%) |
| 4 | Sub minor chain 6 to 7 (FSL above/ near NSL) | 7.62 (25) | Disintegrated rock in bed and sides. |

one chain = 100 feet =30.48 m

TABLE 2.Particle size analysis of bed material of Minor No.3 	   of RMC

| S.No. (1) | Reach (2) | Mechanical composition |  |  |
| --- | --- | --- | --- | --- |
|  |  | Clay % (3) | Silt % (4) | Sand % (5) |
| 1 | Chain 20 | 94% | - | 6% |
| 2 | Chain 45 | 40% | 36% | 24% |
| 3 | Chain 87.50 | 20% | 56% | 24% |
| 4 | Chain 97.50 | 14% | 22% | 64% |
| 5 | Chain 110 | 6% | 30% | 64% |

TABLE 3.Seepage loss study in Minor No. 2 by Ponding method

| S. No. (1) | Reach (2) | Losses in Cumec per M. sq.m.(Cusecs per M.Sq.ft) (3) | Type of strata (4) |
| --- | --- | --- | --- |
| 1 | RD 150 m to 195 (chain 5 to 6.50) | 7.62(25) | Jhinkra in bed, clay in sides. |
| 2 | RD 1170 m to 1200 m (chain 39 to 40) | 5.49(18) | Bed and sides of clay mixed with silt. Minor is in partial cutting and filling. |
| 3 | RD 1530 m to 15543 (chain 51 to 51.23) | 7.32(24) | Jhinkra in bed, clay in sides Reach is upstream of VRB. |
| 4 | RD 2700 m to 2790 (chain 90 to 93 | 2.44(8) | Clay in bed & sides Maximum filling reach. |
| 5 | RD 3600 m to 3660 m (chain 120 to 122) | 7.32(24) | Jhinkra in bed, clay in sides River 180 m away and 11m deep |
| 6 | RD 3990 m to 4020 m (chain 133 to 134) | 7.62(25) | Clay with silt in bed & sides Reach is upstream of VRB. |
| 7 | RD 4200 m to 4230 m (chain 140 to 141) | 12.20(40) | Silt in bed and sides River 115 m away and 10 m deep . |
| 8 | RD 4350 m to 4380 m (chain 145 to 146) | 17.98(59) | Silt in bed & sides. River 190 m away and 10 m deep . |
| 9 | RD 4530 m to 4575 m (chain 151 to 152.50) | 19.51(64) | Silt in bed & sides. River is 225 m away and 9 m deep |
| 10 | R D 5070 m to 5100 m (chain 169 to 170) | 7.92(26) | Silt in bed & sides. Reach is D/S of National Highway. |

TABLE 4. Particle size analysis of bed material of Minor No. 2

| S. No. (1) | Reach (2) | Clay (<0.002 mm) (3) | Silt (0.002 to 0.02 mm) (4) | Sand (0.02 to 2 mm ) (5) |
| --- | --- | --- | --- | --- |
| 1 | Chain 9.50 | 17.45 | 50.6% | 32% |
| 2 | Chain 38 | 90% | - | 10% |
| 3 | Chain 90 | 33% | 63% | 4% |
| 4 | Chain 135 | 20% | 44% | 36% |
| 5 | Chain 159 | 36% | - | - |

# TABLE 5. SALIENT FEATURES OF GUDHA IRRIGATION PROJECT

| S. No. (1) | Items ( 2 ) | Present data ( 3 ) | Proposed data as per modernisation project report ( 4 ) |
| --- | --- | --- | --- |
| 1 | Cross catchment area | 291 sq.mile (753.69 sq.Km) | 291 sq.mile (753.69 sq.Km) |
| 2 | Average annual monsoon rainfall(Average of 50 years) | 28" (700 mm) | 28"(700 mm) |
| 3 | Gross capacity | 2748 M cft (77.81 Mcum) | 3375 M cft. (95.57 Mcum) |
| 4 | Live storage capacity | 2678 Mcft. (75.83 Mcum) | 3305 Mcft. (93.60 Mcum) |
| 5 | Total length of Bund | 7500 ft (2286 m) | 7900 ft (2408 m) |
| 6 | Expected annual flow irrigation in normal year with duty of 8 Acres per M cft. | 21424 Acres (8670 ha) | 26440 Acres (10700 ha) |
| 7 | Top width of earthen bund | 15' (4.50 m) | 15' (4.50 m) |
| 8 | Depth of water above cill | 31'(9.45 m) | 34.50'(10.50 m) |
| 9 | Gross command area | 44807 Acres (18133 ha) | 44807 Acres (18133 ha) |
| 10 | Culturable command area | 25625 Acres (10370 ha) | 26824 Acres (10860 ha) |
| 11 | No. of villages benefitted | 45 Nos | RMC 25 Nos, LMC 20 Nos |
| 12 | No. of minors | 20 | 20 |
| 13 | Intensity of irrigation (a) Rabi (b) Kharif | 60% Nil | 85% } 98.5% 13.50% } |
| 14 | (A) LEFT MAIN CANAL Discharge at head | 112.97 Cusec (3.29 cumec) | 118.57 Cusec (3.36 cumec) |
| 15 | Culturable Commanded Area | 8510 Acres. (3444 ha) | 8708 Acres (3524 ha) |
| 16 | (B) RIGHT MAIN CANAL Discharge at Head | 225 Cusecs (6.37 cumec) | 249.45 Cusecs (7.06 Cumec) |
| 17 | Culturable Commanded Area | 17116 Acres. (6927 Hect.) | 18116 Acres. (7331 ha) |

TABLE 6  SALIENT FEATURES OF COMMAND AREA OF Gudha PROJECT

| S. No. (1) | Item (2) | Total (3) | RMC (4) | LMC (5) | Mr.3 RMC (6) | Mr.2 LMC (7) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | G.C.A. (Acres) | 44807 | 30706 | 14101 | 1436 | 1553 |
| 2 | New CCA (Acres) | 26824 | 18116 | 8708 | 1325 | 114 |
| 3 | New ICA (98.5%) | 26422 | 17844 | 8577 | 1305 | 1131 |
| 4 | OLD CCA | 25626 | 17116 | 8510 | 672 | 1208 |
| 5 | Old ICA@ 60% | 15376 | 10270 | 5106 | 403 | 725 |
| 6 | Difference of New & old CCA | 1198 | 1000 | 198 | 653 | 60 |
| 7 | Difference of New & Old ICA | 11046 | 7574 | 3471 | 902 | 406 |
| 8 | Actual Irrigation (Maximum) | 21438 | 13625 | 7513 | 641 | 1019 |
| 9 | %Actual Irrigation Total Kharif Rabi | 80% 22% 58% | 75% 18% 57% | 86% 26% 60% | 48% 50% 43% | 84% 29% 55% |
| 10 | Discharge(cusecs) new/old | - | 249.45/225.00 | 118.57/112.97 | 15.12/ 8.00 | 14.03/ 5.48 |
| 11 | Length feet/kilometre | - | 92000'/28 Km (old) | 86000'/26 Km (old) | 11700'/ 3.56 Km (new) | 22245'/ 6.78 Km (new) |

TABLE 7. Proposed Design data of Outlets / APM'S  of Minor No. 3 of RMC

| Locations of outlets |  | Details of minors |  |  | C.C.A. of Chaks in Acres | Design discharge of outlet in Cusecs | Type of outlet | Outlet crest width | Designed Water depth over crest (H) | Crest level FSL-H | R.L. of soffit roof block of APM | Minimum modular head in meters | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R.D. in Cha-ins | Out-let no. | Bed level in metres | Full supply depth | Full supply level in meters |  |  |  |  |  |  |  |  |  |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) | (9) | (10) | (11) | (12) | (13) | (14) |
| (A) Open Flume Outlet |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1(R) | 291.71 | 0.50 m (1.65') | 292.21 | 114 | 1.27 | Cast iron open flume with roof block | 0.15 m (0.5') | 0.273 m (0.895') | 291.937 | 292.26 | 0.04 m | Variation gauge with zero at canal designed FSL be fixed at outlet on canal side |
| 17 | 3(L) | 291.67 | 0.50 m (1.652) | 292.17 | 137 | 1.52 | -do- | 0.15 m (0.5') | 0.308 m (1.01') | 291.862 | 292.22 | 0.05 m | -do- |
| 30 | 4(L) | 290.37 | 0.53 m (1.75') | 290.20 | 62 | 0.69 | -do- | -do- | 0.182 m (0.596') | 290.718 | 290.95 | 0.03 m | -do- |
| 50 | 5(R) | 288.67 | 0.49 m (1.60') | 289.16 | 161 | 1.79 | -do- | -do- | 0.343 m (1.125) | 288.817 | 289.21 | 0.05 m | -do- |
| 55 | 6(R) | 288.61 | 0.49 m (1.60') | 289.10 | 147 | 1.63 | -do- | -do- | 0.322 m (1.057') | 288.778 | 289.15 | 0.05 m | -do- |
| 55 | 7(L) | 288.61 | 0.49 m (1.60') | 289.10 | 72 | 0.80 | -do- | -do- | 0.200 m (0.658) | 288.900 | 289.15` | 0.03 m | -do- |
| 80 | 8(L) | 286.64 | 0.30 m (1.0') | 286.94 | 39 | 0.43 | -do- | -do- | 0.133 m (0.435') | 266.807 | 286.99 | 0.02 m | -do- |
| 88 | 9(R) | 285.17 | 0.30 m (1.0') | 285.47 | 51 | 0.57 | -DO- | -DO- | 0.16 m (0.525') | 285.310 | 285.52 | 0.03 m | -do- |
| 117 | Tail (L) | 283.47 | 0.26 m (0.85') | 283.73 | 85 | 0.95 | Masonry open flume | 0.098 m (0.32') | 0.305 m (1.0') | 283.425 | - | 0.045 m | 1.5'(45 cm) gauge with zero at crest level be fixed u/s of the crest |
| 117 | Tail(R) | 283.47 | 0.26 m (0.85') | 283.73 | 51 | 0.57 | -do- | 0.06 m (0.2') | 0.305 m (1.0') | 283.425 | - | 0.045 m |  |
| (B)Adjustable Proportional Module (A.P.M.) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 2(R) | 291.67 | 0.5 m (1.65') | 292.17 | 105 | 1.17 | APM | 0.15 m (0.5') | 0.317 m | 291.853 | 291.975 | 0.09 m | y=0.122 m variation gauge be fixed on outlet canal side |

TABLE 8.  Revised salient features of Minor No. 2 of LMC and  		Minor No. 3 of RMC

| S. No. | Particulars | Minor No.2 of Left Main Canal | Minor No.3 of Right Main Canal |
| --- | --- | --- | --- |
| (1) | (2) | (3) | (4) |
| 1 | Off take point | RD 4663 m (15300 ft) | RD 11857 m (38900 ft ) |
| 2 | Length of minor | 7080 m (23220 ft ) | 3566 m (11700 ft) |
| 3 | Gross commanded area | 620.74 ha (1553.48 Acres) | 581 ha (1436 Acres) |
| 4 | Culturable commanded area | 464.75 ha (1148.38 Acres) | 536 ha (1325 Acres) |
| 5 | Intensity of irrigation | 85% in Rabi & 13.5% in Kharif Total 98.5% |  |
| 6 | Design discharge at head | 0.397 cumec (14.03 cusec) | 0.43 cumec (15.13 cusec) |
| 7 | Bed width | 1.07 metres (3.50 feet) | 1.52 metres (5 feet) |
| 8 | Full supply depth | 0.53 m (1.75 feet) | 0.50 metres (1.65 feet ) |
| 9 | Water surface slope | 1 in 2000 | 1 in 2500 |
| 10 | No. of outlets | 19 | 11 |

TABLE 9.Revised  design data of Minor No.3 of Right Main Canal          as per Intervention

| S. No. | Reach in Metres (feet) | Design dischar-ge in cumec (cusec) | Velocity in m/sec (Ft/sec) | Bed width in metre (Feet) | Full supply depth in metre (feet) | Water surface slope |
| --- | --- | --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) |
| 1 | 0-518 m (0-1700') | 0.43 (15.13) | 0.44 (1.44) | 1.52 (5') | 0.50 (1.65) | 1 in 2500 |
| 2 | 518-914 m (1700-3000') | 0.31 (11.07) | 0.41 (1.36) | 0.91 (3') | 0.53 (1.75') | 1 in 2500 |
| 3 | 914-1676 m (3000-5500') | 0.20 (7.01) | 0.37 (1.21) | 0.61 (2') | 0.49 (1.60') | 1 in 2500 |
| 4 | 1676-2774 m (5500-9100') | 0.08 (2.67) | 0.29 (0.96) | 0.61 (2') | 0.30 (1.0') | 1 in 2500 |
| 5 | 2774-3566 m (9100- 11700') | 0.045 (1.60) | 0.26 (0.85) | 0.45 (1.5) | 0.26 m (.85') | 1 in 2500 |

TABLE 10. Design data of Minor No.2 of LEFT Main Canal

| Existing design data |  |  |  |  |  | Proposed design data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reach in metre | Desi- gn disc- harge in cumec (cusec) | Velo-city in m/sec (Ft/sec) | Bed in metre (Ft) | Full supp-ly depth in m (Ft) | Water surf-ace slope | Reach in metre | Desi-gn disc- harge in cumec (cusec) | Velo-city in m/sec (Ft/sec) | Bed width in metre (Ft) | Full supp-ly depth in m (Ft) | Water surf-ace slope |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) | (9) | (10) | (11) | (12) |
| 0-120 | 0.155(5.48) | 0.26 (o.85) | 0.91 (3') | 0.50m (1.65') | 1 in 5000 | 0-800m | 0.397 (14.03) | 0.47m (1.56) | 1.07m(3.50') | 0.53m (1.75') | 1 in 2000 |
| 120-990 | 0.155(5.48) | 0.36 (1.19) | 0.91m (3') | 0.38m(1.25) | 1 in 5000 | 800-1560 | 0.348 (12.29) | 0.46 (1.52) | 0.91 (3ft) | 0.53 (1.75ft) | 1 in 2000 |
| 990-1560 | 0.14 (4.97) | 0.36 (1.19) | o.91 (3ft) | 0.36 (1.20ft) | 1 in 2000 | 1560-1860 | 0.323 (11.42) | 0.65 (2.14') | 0.91m (3') | 0.40 (1.30') | 1 in 750 |
| 1560- 1680 | 0.13 (4.46) | 0.49 (1.63) | 0.61m(2') | 0.33m(1.10') | 1 in 750 | 1860-2900 | 0.305 (10.78) | 0.45 (1.48) | 0.91m (3') | 0.50m(1.65) | 1 in 2000 |
| 1680- 2280 | 0.13 (4.46 | 0.29 (0.94) | 0.91 (3') | 0.41m (1.35) | 1 in 3500 | 2900-4200 | 0.251 (8.87) | 0.43 (1.41) | 0.91 (3') | 0.44m (1.45) | 1 in 2000 |
| 2280- 3360 | 0.11(4.01) | 0.28 (0.91) | 0.91 (3') | 0.38 (1.25) | 1 in 3500- | 42000-4760 | 0.189 (6.69) | 0.40 (1.32) | 0.76 (2.5) | 0.43 (1.400 | 1 in 2000 |
| 3360- 4140 | 0.09 (3.19) | 0.16 (0.54) | 0.76 (2.5') | 0.38 (1.25) | 1 in 3500 | 4760-5070 | 0.156 (5.51) | 0.38 (1.25) | 0.76 (2.5) | 0.38m (1.25) | 1 in 2000 |
| 4140- 5040 | 0.07 (2.58) | 0.25 (0.83) | 0.61 (2') | 0.36 (1.20) | 1 in 3500 | 5070-6180 | 0.12 (4.22) | 0.36 (1.17) | 0.76 (2.5) | 0.33m (1.10') | 1 in 2000 |
| 5040-5670 | 0.05 (1.80) | 0.23 (0.75) | 0.61 (2.0') | 0.30 (1.0') | 1 in 3500 | 6180-7080 | 0.06 (1.74) | 0.28 (0.93) | 0.61m (2') | 0.23m (0.75) | 1 in 2000 |

TABLE 11.  Revised command area of minor no.3 of R.M.C. as per   intervention

| Distance from head in metre | Outlet No. | Left or right | G.C.A. in Acres | C.C.A. in Acres |
| --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) |
| 396 m | 1 | Right | 120 | 114 |
| 518 m | 2 | Right | 107 | 105 |
| 518 m | 3 | Left | 176 | 137 |
| 884 m | Sub minor | Left | 313 | 301 |
| 914 m | 4 | Left | 67 | 62 |
| 1524 m | 5 | Right | 179 | 161 |
| 1676 m | 6 | Right | 167 | 147 |
| 1676 m | 7 | Left | 74 | 72 |
| 2433 m | 8 | Left | 44 | 39 |
| 2682 m | 9 | Right | 52 | 51 |
| 3566 m | 10 | Tail(R/s) | 52 | 51 |
| 3566 m | 11 | Tail(L/S) | 89 | 85 |

1436 Acres   1325 Acres

TABLE 12 . Revised command area of Minor No.2 of L.M.C.  as per  intervention

| S. No. | Distance from head in metre | Outlet No. | Left or right | Name of village | G.C.A. in Acres | C.C.A. in Acres |
| --- | --- | --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) |
| 1 | 463 m | 1 | Left | Raghunathpura | 66.18 | 60.76 |
| 2 | 463 m | 1 | Right | -do- | 80.86 | 79.70 |
| 3 | 954 m | 2 | -do- | Mangli kalan | 87.42 | 63.76 |
| 4 | 1836 m | 3 | -do- | -do- | 90.20 | 53.36 |
| 5 | 2100 m | 2 | Left | -do- | 49.50 | 47.60 |
| 6 | 2752 m | 3 | -do- | -do- | 131.32 | 105.22 |
| 7 | 3094 m | 4 | Right | Mangli khurd | 93.16 | 52.60 |
| 8 | 3193 m | 4 | LEFT | -do- | 156.28 | 80.62 |
| 9 | 4152 m | 5 | Right | -do- | 51.24 | 40.04 |
| 10 | 4435 m | 5 | LEFT | -do- | 112.70 | 96.82 |
|  | 5040 m | Road bridge on National Highway |  | Sub total | 918.86 | 680.48 |
| 11 | 5055 m | 6 | Right | Mangli khurd | 53.70 | 51.22 |
| 12 | 5070 m | 6 | Left | Mangli khurd Chatarganj | 93.70 | 60.62 |
| 13 | 5573 m | 7 | Right | Bichari | 70.14 | 65.44 |
| 14 | 5573 m | 8 | -do- | -do- | 46.76 | 46.54 |
| 15 | 5608 m | 7 | Left | Chatarganj | 156.16 | 55.16 |
| 16 | 5850 m | 8 | -do- | Bichari | 44.72 | 41.86 |
| 17 | 6442 m | 9 | Right | Bichari | 47.02 | 43.70 |
| 18 | 6690 m | 9 | Left | -do- | 57.56 | 53.82 |
| 19 | 7080 m | Tail | - | -do- | 65.22 | 49.54 |

TABLE 13. Existing pucca structures across the Minor no.2 and                   calculated head loss

| S. No. | R.D. in metre | Name and size of structure | Design discharge in cusecs | Desig-ned B.W. | Designed FSD | Veloc-ity ft/se-c | Head losses |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) |
| 1 | 0 | Head regulator 2' dia pipe | 14.03 (0.397 cumec) | - | - | - | 15 cm |
| 2 | 33 | VRB 4' wide incomplete | 14.03 | 3.50' | 1.75' | 1.56 | 3 cm |
| 3 | 45 | Flume 4'wide | 14.03 | 3.50' | 1.75' | " | 3 cm |
| 4 | 120 | Fall 3' wide drop 0.20 m | 14.03 | 3.50' | 1.75' | " | - |
| 5 | 465 | Fall 3' wide drop 0.50m | 14.03 | 3.50' | 1.75' | " | - |
| 6 | 770 | VRB 4' wide (Incomplete) | 14.03 | 3.50' | 1.75' | " | 3 cm |
| 7 | 800 | Fall (fully destroyed drop 0.06 m | 14.03/ 12.29 | 3.50'/3.0' | 1.75'/ 1.75 | 1.56/ 1.52 | _ |
| 8 | 1140 | VRB 2'9"wide | 12.29 | 3' | 1.75' | 1.52' | 5 cm |
| 9 | 1560 | VRB 1'6"wide | 12.29/ 11.42 | 3'/3' | 1.75/ 1.30 | 1.52/ 2.14 | 24 Cm |
| 10 | 1750 | VRB 3' wide (Incomplete) | 11.42 | 3' | 1.30' | 2.14 | 6 cm |
| 11 | 1860 | Fall 2' wide (Incomplete) | 11.42/ 10.78 | 3'/3' | 1.30/ 1.65 | 2,14/ 1.48 | 12 cm |
| 12 | 1960 | VRB 2' wide (Incomplete) | 10.78 | 3' | 1.65' | 1.48 | 8 cm |
| 13 | 2280 | VRB 2.5'wide | 10.78 | 3' | 1.65' | 1.48 | 5 cm |
| 14 | 2900 | VRB 2.5'wide | 10.78/ 8.87 | 3'/ 3' | 1.65'/ 1.45' | 1.48/ 1.41 | 5 cm |
| 15 | 4080 | VRB 1.5' dia pipe | 8.87 | 3' | 1.45' | 1.41 | 19 cm |
| 16 | 4440 | Pedestrian crossing (1' dia pipe) | 6.69 | 2.5' | 1.40' | 1.32 | 55 cm |
| 17 | 4760 | Fall cum VRB (1'dia pipe) | 6.69/ 5.51 | 2.5'/2.5' | 1.40'/ 1.25' | 1.32/ 1.25 | 38 cm |
| 18 | 5040 | DRB (1.0' dia pipe) | 5.51 | 2.5' | 1.25' | 1.25' | 38 CM |

Table 14. COMMAND STATEMENT OF MINOR NO. 2 OF LEFT MAIN CANAL AS PER INTERVENTION

| S. No. | Chak No./outlet No. | Chainag-e | Critical N.S.L. In chak |  | Length of W.C. from outlet to critica-l NSL in Km. | F.S.L./ WL in field at naka point (NSL+0.15 cm) | Water slope of WC 0.25%0 1/4000 | FSL/WL require-d at head. 6+0.25 x 5 | FSL in Disty/ minor require-d at outlet 8+0.3 | F.S.L. in Disty/ minor at outlet as per old L-section | Positio-n of existing FSL lower(-) or Higher (+) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | N.S.L. | Locatio-n Khas -ra No. |  |  |  |  |  |  |  |
| (1) | (2) | (3) | (4 a) | (4 b) | (5) | (6) | (7) | (8) | (9) | (10) | (11) |
| 1 | 1 R | 455 | 294.65 | 493 | 0.420 | 294.80 | 0.25%0 | 294.8+0.25x.42=294.905 | 295.20 | 294.37 | -0.83 |
| 2 | 1 L | 455 | 294.045 | 420 | 0.255 | 294.195 | 0.25%0 | 294.26 | 294.56 | 294.37 | -0.19 |
| 3 | 2 R | 954 | 292.695 | 15 | 0.00 | 292.845 | 0.25 | 292.845 | 293.149 | 292.87 | -0.279 |
| 4 | 3 R | 1836 | 290.915 | 178 | 0.00 | 291.065 | 0.25 | 291.065 | 291.365 | 291.37 | - |
| 5 | 2 L | 2100 | 290.76 | 117 | 0.00 | 290.91 | 0.25 | 290.91 | 291.21 | 290.95 | -0.26 |
| 6 | 3 L | 2752 | 289.37 | 1248 | 0.600 | 289.52 | 0.25 | 289.67 | 289.97 | 290.62 | +0.65 |
| 7 | 4 R | 3094 | 290.365 | 290 | 0.42 | 290.515 | 0.25 | 290.62 | 290.92 | 290.34 | -0.58 |
| 8 | 4 L | 3193 | 290.56 | 279 | 0.18 | 290.71 | 0.25 | 290.755 | 291.055 | 290.30 | -0.755 |
| 9 | 5 R | 4150 | 290.03 | 104 | 0.135 | 290.18 | 0.25 | 290.22 | 290.52 | 289.52 | -1.000 |
| 10 | 5 L | 4435 | 290.365 | 41/150 | 0.51 | 290.515 | 0.25 | 290.64 | 290.94 | 289.68 | -1.26 |
| 11 | 6 R | 5055 | 289.85 | 127/195 | 0.27 | 289.00 | 0.25 | 289.064 | 289.367 | 288.85 | -0.517 |
| 12 | 6 L | 5070 | 288.885 | 124/181 | 0.00 | 289.035 | 0.25 | 289.035 | 289.335 | 288.84 | -0.495 |
| 13 | 7 R | 5573 | 288.44 | 150 m from head | 0.15 | 288.59 | 0.25 | 288.627 | 288.927 | 287.79 | -1.137 |
| 14 | 7 L | 5573 | 287.51 | 1333/ 1530 | 0.09 | 287.66 | 0.25 | 287.68 | 287.98 | 287.78 | -0.20 |
| 15 | 8 R | 5608 | 287.65 | 779 | 0.12 | 287.80 | 0.25 | 287.83 | 288.13 | 287.79 | -0.34 |
| 16 | 8 L | 5850 | 287.285 | 776 | 0.00 | 287.435 | 0.25 | 287.435 | 287.735 | 287.655 | -0.08 |
| 17 | 9 R | 5442 | 286.925 | 795 | 0.18 | 287.075 | 0.25 | 287.12 | 287.42 | 286.66 | -0.76 |
| 18 | 9 L | 6690 | 286.385 | 111/ 1210 | 0.24 | 286.535 | 0.25 | 286.595 | 286.895 | 286.52 | 0.375 |
| 19 | 10 T | 7080 | 285.845 | 17 | 0.24 | 285.995 | 0.25 | 286.055 | 286.355 | 286.34 | 0.015 |

Table 15. REVISED COMMAND STATEMENT OF MINOR NO. 3 OF RIGHT MAIN CANAL AS PER INTERVENTION

| S. No. | Chak No./ outlet No. | chain-age | Critical NSL in the chak |  | Length of W.C. from outlet to critical NSL in Km. | FSL/WL in field/ Naka point (NSL + 0.15 m) | Bed slope of W.C. 0.25% (1 in 4000) | FSL/WL requi- red at head 6+ 0.25 x 5 | FSL in Disty./ minor required at outlet (8+0.30) | FSL in Disty./ minor at outlet as per old L-section. | Position of exis- ting FSL lower (-) or Higher(+) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | N.S.L. | Location khasra No. |  |  |  |  |  |  |  |
| (1) | (2) | (3) | (4 a) | (4 b) | (5) | (6) | (7) | (8) | (9) | (10) | (11) |
| 1 | 1 (R) | 13 | 292.19 | 1198 | 338 m | 292.34 | 0.09 | 292.43 | 292.73 | > 292.21 | -0.52 m |
| 2 | 2 (R) | 17 U.S. | 291.23. | 1696 | 523 m | 291.38 | 0.13 | 291.51 | 291.81 | < 292.17 | +0.36 m |
| 3 | 3 (L) | 17 U.S. | 292.57 | 918 | 161 m | 292.72 | 0.04 | 292.76 | 293.06 | > 292.17 | -0.89 m |
| 4 | 4 (L) | 30 U/S | 291.30 | 1253 | 805 m | 291.45 | 0.20 | 291.65 | 291.95 | > 290.87 | -1.05 m |
| 5 | 5 (R) | 50 | 289.52 | 2592 | 242 m | 285.67 | 0.06 | 289.73 | 290.03 | > 289.16 | -0.87 m |
| 6 | 6 (R) | 55 U/S | 288.52 | 1641 | 49 m | 288.67 | 0.01 | 288.68 | 288.98 | < 289.10 | +0.12 m |
| 7 | 7(L) | 55 U/s | 288.58 | 1637 | 362 m | 288.73 | 0.09 | 288.82 | 289.12 | > 289.10 | -0.02 m |
| 8 | 8(L) | 80 | 286.15 | 2414 | 201 m | 286.30 | 0.05 | 286.35 | 286.65 | < 286.94 | +0.29 m |
| 9 | 9 (R) | 88 | 285.39 | 1537 | 443 m | 285.54 | 0.11 | 285.65 | 285.95 | > 285.48 | -0.47 m |
| 10 | Tail (Right) | 117 D/S | 283.55 | 1239 | 423 m | 283.70 | 0.10 | 283.80 | 284.10 | > 284.13 | +0.03 m |
| 11 | Tail (Left) | 117 D/S | 283.69 | 2465 | 101 m | 283.84 | 0.02 | 283.86 | 284.16 | > 284.13 | +0.03 m |

TABLE 16.  COMMAND STATEMENT OF SUB MINOR OF MINOR NO. 3 OF RIGHT MAIN CANAL AS PER INTERVENTION

| S. No. | Chak No./outlet No. | Chainag-e | Critical NSL in the chak |  | Length of W.C. to critical NSL from outlet in Km. | FSL/WL in field/ Naka point (NSL + 0.15 m) | Bed slope of W.C. 0.25% (1 in 4000) | FSL/WL requi- red at head 6+0.25x 5 | FSL in Disty./ minor requir-ed at outlet (8+0.30 | FSL in Disty./ minor at outlet as per old L-section. | Position of exis- ting FSL lower (-) or Higher(+) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | N.S.L. | Location khasra No. |  |  |  |  |  |  |  |
| (1) | (2) | (3) | (4 a) | (4 b) | (5) | (6) | (7) | (8) | (9) | (10) | (11) |
| 1 | 1(L)(SM) | 3 | 289.625 | 1256 | 0.4 | 289.775 | 0.25% | 289.875 | 290.175 | 290.52 | FSL is higher |
| 2 | 3(R) (SM) | 25 | 288.085 | 1621 | 0.2 | 288.235 | 0.25% | 288.285 | 288.585 | 287.92 | FSL is lower by 0.665 m |
| 3 | 2(L)(SM) | 25 | 287.525 | 1320 | 0.3 | 287.675 | 0.25% | 287.75 | 288.05 | 287.92 | FSL is lower by 0.13 m |
| 4 | Tail(SM) | 60 | 285.175 | 1409 | 0.0 | 287.325 | 0.25% | 285.325 | 285.625 | 285.50 | FSL is lower by 0.125 m |

Director (retired), Minor Irrigation Schemes, Jaipur (Rajasthan:India), ycagrawal@yahoo.com

Phone +91(141)2203508 , Mobile +919414056550

Key words: water management, micro net work survey, vertical falls, soil survey, seepage losses,