### Annexure 2

### HYDROLOGY

# Basic latest Data/ Information Required for Hydrological Design

- General: Brief History of the Project giving salient features related to the Dam & spillway.

- Data  Required

- Maps & Plans : Catchment Area Plans Scale 1:50,000 or 1:25,000  showing contour lines

- River course & tributaries to be shown with their names

- Location of all gauge & discharge sites

- Location of water storage structures intercepting the catchment

- Location of ordinary & SRRG raingauge stations in & around the vicinity of the catchment of dam site / G.D. site

- Location of nearby minor medium and major dams

I - FLOOD STUDIES

Following basic data are required for flood computation studies.

- Data Required

- Critical Flood Events: Rain fall & corresponding run off data for three or four flood events with date & time experienced in the catchment.

- Hourly gauge /discharge data at the nearest G & D station preferably on the same side of ridge

- Corresponding hourly rainfall data of rain gauge station including data & charts of SRRG located in the catchment or in the close vicinity of it

- If observed short duration data are not  available,  then respective sub zone reports published by CWC is to be referred & used

- Generalized  P.M.P. (Probable Maximum Precipitation) Atlas for those river basins for which CWC have prepared & published

- Type of strata of bed and sides of Nalla /River

- Other Miscellaneous data like silt load etc.

###### DRAWINGS

- L – Section of longest stream in the catchment for computing equivalent slope

- The suitable cross sections upstream and downstream of proposed dam axis including L- section with observed HFL marked with time & date

# II -  YIELD STUDIES:

Following basic data are required for Yield Studies

- Rainfall Data.

- Monthly rainfall data for all the rain gauge stations in and around the catchment for minimum 25 years.

## Run off Data

- Monthly Runoff Data at the location nearest gauge & discharge sites for maximum available period

- Monthly average data of all the water storage structures intercepting the catchment

- Monthly volumes of out flows from spillways of intercepting storage structures

- Monthly data of storage/spillway in nearby two tanks for comparison of data

- STEPS FOR COMPUTATION OF YIELD BASED ON OBSERVED DISCHARGE DATA AT  G & D SITE :

- Mark the location of proposed Dam site on G. T. sheet.

- Mark the Catchment Area on G.T sheet  & measure it by Planimeter or by any other method.

- Mark rain gauge stations in and around the catchment & construct “Thiessen’s polygon” by drawing perpendicular bisectors to the lines joining rain gauge stations.

- Measure effective catchment area of each rain gauge

- Calculate influence factor for each rain gauge station

Effective Catchment Area

Influence Factor  = -------------------------------------------

Total Catchment Area

- Collect monthly rainfall data (for monsoon period) of the considered rain gauge stations for minimum 25 – 30 years

- Arrange rainfall data month wise for monsoon period i.e. from June to October

- Compute weighted rainfall by multiplying monthly rainfall with Influence Factor

- Compute month wise total weighted rainfall (Pd)) in the catchment.

- Select nearby G&D site where discharge data for minimum 25-30 years are available.

- Collect runoff data (Rg) at G&D site for the months June to Oct.

- Compute month wise total weighted rainfall at G&D site (Pg) by repeating step a to i

- Compute month wise linear relation between weighted rainfall (Pg) & Runoff (Rg) at G&D Site by using “Linear regression analysis”

Rg = APg +B

Here A and B are coefficients, which are given by

N((∑Pg.Rg)- (∑Pg) ((∑Rg)

A = --------------------------------------

# N (∑Pg.2) – (∑Pg)2

(∑Rg) - A(∑Pg)

B = --------------------------------------------

N

Where N = Number of observation sets between Rg & Pg

Then correlation coefficient ‘R’ is calculated as

N(∑Pg.Rg)- (∑Pg) (∑ Rg)

R     =   _________________________________________

_____________________________________

√[N(∑Pg2)- (∑Pg)2]x [N(∑ Rg2)- (∑Rg)2]

‘R’ lies between 0 to + 1 as Rg can have only positive correlation with Pg.

It indicates good correlation if 0.6 < R < 1.0.

- Compute month wise linear relation between observed weighted rainfall at G&D site (Pg) and observed weighted rainfall at dam site (Pd)

Pd= CPg+ D

# Compute modified total weighted rainfall at dam site (Pm) using correlation computed in step n for each month

# Pm= CPg+D

- Compute runoff at dam site (Rd) using modified total rainfall at dam site (Pm) and correlation computed step n for  each month .

Rd= A Pm + B

# Convert runoff (Rd) to volume in cubic unit

Yield (Y) = Rd * Catchment area

- Arrange yield (Y) in descending order

- Compute probability of every year from (N+1) years, where N is number of years for which runoff data are available.

- Considering yield at 50% dependability, the yield is calculated using formula. M/(N+1).x 100