# Data Required for Design of Minor Irrigation Projects

Whenever a problem of design of Minor Irrigation Project is to be dealt, following information should be collected.

- Geological data :

Geological investigation may be done for the suitability of foundation, water tightness of the reservoir and depth of cut off trench.

- Hydrological data :

The hydrology should be prepared for 50 years return period flood and got vetted by the Chief Engineer ID&R Jaipur. The data should be collected as per Annexure 2.

- Miscellaneous data:

- Probable life of reservoir

- Proposed full tank level and maximum Water Level

- Maximum wind velocity for determination of wave height

- Minimum draw down level/canal sill level

- Calculations for tail water rating curve

- Requirements for roadways

- Capacity and elevations of  sluice

- Chemical analysis of river water and ground water

- Length of dam, non overflow and over flow

- Type of dam:  earthen, masonry etc. to be provided

- Details of saddle dam, if any

.

- Soil tests for borrow area and foundation soils:

Adequate number of soil samples should be got tested in Material Testing Laboratory of M.T. Dn. Jaipur. Samples from each borrow pit at every 0.30 m depth should be collected. For foundation, the soil samples be collected along proposed dam line for every one-metre depth at least upto 3m depth or till the suitable strata is reached. On each side of nalla / river, samples from foundation soil should be taken at locations maximum, middle and minimum height of dam embankment.

Soil tests to be conducted for soil samples are as below:

- Mechanical analysis

- Liquid limit,  plastic limit and shrinkage limit

- Proctor’s Maximum Dry Density and Optimum Moisture Content

- Free swell index

- Specific gravity

- Permeability

- Triaxial shear test (C and )

- Total soluble salts

- Dispersibility

- Sulphate, Carbonate, Organic matter

- List of drawings :

- Index map on a scale of 1:50,000 or 1:25,000 showing location of project and location of existing works affecting/ being affected by proposed development

- Submergence area plan at contour interval not more than 0.5 m on a scale preferably 1:4000 and should cover elevation of at least 2 m above the proposed M.W.L. at dam site

- General lay out plan covering works area in the vicinity of dam site with a contour interval of not more than 0.5 m and preferably on a scale 1: 4000

- Capacity and submergence curve

- Block level plan of dam site

- Tail water rating curve (Elevation discharge curve) along with high flood level

- L-section of river 0.5 km upstream and 1.5 km downstream of proposed dam site

- Cross section of river at axis of dam site, one at upstream and three at downstream    upto 3 m above H.F.L. on both sides and indicating highest flood levels during floods received so far

- Borrow area plan showing location and description of characteristics of proposed material to be used in the construction of the dam including, soil, sand, gravel etc.

- Borrow area plan indicating

- Depth and size of borrow pits/ area

- Quantity of earth available in each borrow pit/area

- Log of exploration of borrow pit/area indicating type of soils available at different depth

- Soil test results of soil of each borrow area

## Additional data required for  remedial measures for existing/ breached dam

- Design of existing/ breached dam

- Shortcomings of existing/ breached dam

- Technical data of existing/ breached dam

- Details of adjoining structures i.e. canal head regulator, spillway, byewash etc.

- Soil properties of existing dam

## Salient Features of _____________________________Minor Irrigation Project

| S. No. | Item | F.P.S. | M.K.S. |
| --- | --- | --- | --- |
| 1. | Gross Catchment Area |  |  |
| 2. | # Intercepted Catchment Area |  |  |
| 3. | # Free Catchment Area |  |  |
| 4. | F.T.L. |  |  |
| 5. | M.W.L. |  |  |
| 6. | T.B.L. |  |  |
| 7. | # Free Board |  |  |
| 8. | # Flood Lift |  |  |
| 9. | # Location # Latitude # Longitude |  |  |
| 10. | # Average annual rainfall of last 50 years |  |  |
| 11. | # Expected annual run off |  |  |
| 12. | # Gross storage capacity |  |  |
| 13. | Dead storage capacity |  |  |
| 14. | Live storage capacity |  |  |
| 15. | # Total length of Dam |  |  |
| 16. | # Top width of earthen dam |  |  |
| 17. | Design discharge of spillway |  |  |
| 18. | # Length of waste weir /spillway |  |  |
| 19. | Maximum height of Dam |  |  |
| 20. | # U/S & D/S slope of Dam |  |  |
| 21. | Sill level |  |  |
| 22. | Gross command area |  |  |
| 23. | Culturable command area |  |  |
| 24. | No. of villages benefited |  |  |
| 25. | Submergence area at F.T.L. (ha) |  |  |
| 26. | Intensity of Irrigation |  |  |
| 27. | Cost of project |  |  |
| 28. | Cost per hectare |  |  |
| 29. | B.C. Ratio |  |  |
| 30. | Difference between U/S MWL & D/S tail water level |  |  |
| 31. | Duty |  |  |
| 32. | Water allowance at outlet head |  |  |
| 33. | Design discharge of canal at head |  |  |
| 34. | Length of Main Canal |  |  |